// Fixed ultrasonic perimeter alarm — HC-SR04 -> Arduino

const int TRIG_PIN = 2;

const int ECHO_PIN = 3;

const int BUZZER_PIN = 8;

const int LED_PIN = 13;

const unsigned int SAMPLE_COUNT = 5;    // median/avg smoothing

const float THRESH_CM = 200.0;          // 200 cm = 2 meters

const unsigned long ALARM_ON_MS = 100;  // buzzer on pulse length

const unsigned long ALARM_REPEAT_MS = 300; // repeat spacing while object present

void setup() {

pinMode(TRIG_PIN, OUTPUT);

pinMode(ECHO_PIN, INPUT);

pinMode(BUZZER_PIN, OUTPUT);

pinMode(LED_PIN, OUTPUT);

digitalWrite(TRIG_PIN, LOW);

digitalWrite(BUZZER_PIN, LOW);

digitalWrite(LED_PIN, LOW);

Serial.begin(9600);

delay(50);

}

float readDistanceCM() {

unsigned long durations[SAMPLE_COUNT];

for (unsigned int i=0; i<SAMPLE_COUNT; ++i) {

// trigger pulse

digitalWrite(TRIG_PIN, LOW);

delayMicroseconds(2);

digitalWrite(TRIG_PIN, HIGH);

delayMicroseconds(10);

digitalWrite(TRIG_PIN, LOW);

unsigned long dur = pulseIn(ECHO_PIN, HIGH, 30000UL); // timeout 30 ms

if (dur == 0) dur = 30000UL; // treat timeout as long distance

durations[i] = dur;

delay(10);

}

// simple average but ignoring the largest and smallest could be better

unsigned long sum = 0;

for (unsigned int i=0;i<SAMPLE_COUNT;i++) sum += durations[i];

unsigned long avg = sum / SAMPLE_COUNT;

float distCm = (avg / 58.0); // microseconds->cm

return distCm;

}

void beepOnce() {

digitalWrite(LED_PIN, HIGH);

digitalWrite(BUZZER_PIN, HIGH);

delay(ALARM_ON_MS);

digitalWrite(BUZZER_PIN, LOW);

digitalWrite(LED_PIN, LOW);

}

void loop() {

float d = readDistanceCM();

Serial.print("Distance (cm): ");

Serial.println(d);

if (d > 0.0 && d <= THRESH_CM) {

// object detected in restricted zone

beepOnce();

delay(ALARM_REPEAT_MS);

// keep beeping until object gone (loop will continue)

} else {

// idle: short delay to reduce sensor spam

delay(80);

}

}